# Practical 2 – Topic 2

## Sag of a meter rule

This experiment can also be a ‘design lab’ if you give no further information other than listing some of the materials to be used.

### Criteria assessed

- DCP

- CE

### Materials needed

- Meter rule (plastic or wooden)

- Vertical stands

- Clamps

- Ruler

- Weights

### What to do

Place a plastic or wooden meter rule across two supports and hang a mass from its middle point. Measure the distance y by which the rule bends at the middle. Repeat with other masses.

- How does y depend on the hanging mass m?

Theory suggests that.

- How should the variables be plotted so that a staight line is obtained and p can be found?

You can modify this experiment by clamping the meter rule on a horizontal table so that a length L of the rule extends beyond the edge of the table. You can then attach weights to a specific fixed point on the rule and measure the deflection y of the rule from its equilibrium position. How is y related to the attached mass m this time?

m

y