# Mark scheme for Extension Worksheet – Topic 5, Worksheet 2

1	If  is open no current leaves the battery and so the power is 0 W. So  must be closed for any power to be dissipated. If  is closed and  is open, the current only goes through resistor X and so the power is 24 W. If  is closed and  is also closed then current goes through X and Z and both are in parallel with 12 V so the total power is 48 W.  If  is open and  is closed then current goes through all three resistors. Z is in parallel with 12 V so its power is 24 W. Each of X and Y have 6.0 V across them and the power in each is one fourth of the normal i.e. 6.0 W each. This gives a total of 36 W. Finally, if  and  are both open, current goes through X and Y only and each is at 6.0 V and so the power in each is 6.0 W for a total of 12 W.

| S1 | S2 | S3 | Power/W |
| --- | --- | --- | --- |
| open | closed | open | 0; |
| closed | closed | open | 24; |
| closed | closed | closed | 48; |
| closed | open | closed | 36; |
| closed | open | open | 12; |

[5]

2	The reading will be the emf ; because there is no current in the circuit the resistance does not drop the voltage so effectively one side of the voltmeter is connected to the 0 V line of the battery and the other side to the  V line, with a difference therefore of 6.0 V.	[2]

3	a	3.0 V		[1]

b	As temperature increases the resistance  of the thermistor decreases; so the voltage across it decreases; and hence the voltmeter reading increases.	[3]