# Extension Worksheet – Topic 7, Worksheet 1

1	A natural gas power station has a power output of 350 MW and an efficiency of 45%. Calculate the mass of natural gas that is burned per second given that the energy density of natural gas is 58 MJ kg–1.	[3]

2	A solar cell has a collecting area of 3.0 cm2. When light of intensity 950 W m–2 is incident on the cell the power output from the cell is 40 mW. Estimate the efficiency of the solar cell.	[2]

3	A house needs 3.0 kW of power in order to provide warm water in the bathrooms. The average solar intensity at the position of the house is 720 W m–2 and the efficiency at which the solar panels convert solar energy to thermal energy is 25%. Calculate the area of the solar panel needed.	[3]

4	A hydroelectric power station produces 85 MW of electrical power. The overall efficiency of the station is 35%. The average height of the water centre of mass above the turbines is 58 m.

a	Calculate the rate at which water flows through the turbines of the power station in m3 s–1. (The density of water is 1.0 × 103 kg m–3.)	[3]

b	State two reasons why the efficiency of the power station is less than 100%.	[2]

5	Show that the maximum energy that can theoretically be extracted from a wind turbine of blade radius R when the wind speed is v is  where  is the density of air.	[3]

6	Explain why the actual power extracted from the wind is less than the expression you just derived in the previous problem.	[1]

7	Describe the operation of an oscillating water column (OWC).	[3]

8	The average albedo of the Earth is about 0.30. Explain why an average albedo is quoted.		[1]

9	State two factors that affect the albedo of the Earth.	[2]

10	The power emitted by the Sun is 3.9 × 1026 W and the Sun–Earth distance is 1.5 × 1011 m.

a	Calculate the solar intensity at the upper atmosphere.	[2]

b	Explain why the average intensity of the solar radiation at the Earth’s surface is less than your answer to a.	[2]

c	The average intensity incident on the Earth’s surface is 245 W m–2. Calculate the average Earth temperature that is consistent with this amount of incident radiation assuming that the Earth surface behaves as a black body.	[2]

d	The actual Earth temperature is higher than the answer to c. Suggest a reason for this observation.	[2]

11	On a particular day, the intensity of the solar radiation at the Earth’s surface on the equator is 850 W m–2. Explain why the intensity in locations north and south of the equator will be less than 850 W m–2.	[2]

12	The intensity of the solar radiation at a point in the upper atmosphere facing the Sun is S. The albedo of the Earth is . Explain why the average solar intensity incident on an arbitrary point on the Earth’s surface is .	[3]

13	The average solar intensity on the Earth’s surface is  where the average Earth albedo is  = 0.30 and S = 1400 W m–2. Assuming the Earth surface to behave like a black body, estimate the Earth surface temperature listing any other assumptions you make.		[3]

14	The diagram shows a black body of temperature T1 emitting radiation towards a gray body of lower temperature T2 and emissivity e. No radiation gets transmitted through the gray body.

Using all or some of the symbols T1, T2, e and  state expressions for the intensity:

a	radiated by the black body.	[1]

b	radiated by the gray body.	[1]

c	absorbed by the gray body.	[1]

d	reflected by the gray body.	[1]

15	The black and the gray bodies in question 14 receive as much energy as they lose. Deduce that their temperatures must be the same.	[2]

16	The volume expansion coefficient of water is 2.1 × 10–4 K–1. Estimate the increase in sea level for a temperature increase of 1.5 K of sea water. Assume that the depth of water that is heated is 50 m. List any other assumptions you have made to reach your answer.		[3]

17	A rise in sea level is an effect of global warming that might come about as a result of land-based ice melting.

a	It is estimated that when the West Antarctic ice sheet melts the expected rise in sea level, will be 6 m. Estimate the volume of the West Antarctic ice sheet by assuming that the area of the oceans is 4 × 108 km2. The density of ice is 900 kg m–3 and that of water is 1000 kg m–3.	[2]

b	Explain why ice floating in water will not result in a rise in sea level.	[2]