# Practical 1 – Topic 7

## Boltzmann's radiation law

This lab is a modified version of lab 4F in Physics by Experiment, by J. R. L. Hartley and D. L. Misell, published by Stanley Thornes, 1987.

### Criteria assessed

- DCP

- CE

### Materials needed

- Lamp (with tungsten filament rated at approximately 12 V, 24 W)

- Power supply (0–12 V)

- Variable resistor (0–100 )

- Ammeter and voltmeter

### What to do

- Set up the circuit below:

- Measure the resistance of the lamp, making sure that the current through it is so small that the lamp does not glow.

- The resistance is given by the ratio of voltage to current. The resistance should now be larger than the cold resistance (why?).

- To calculate the temperature, assume that R = R0(1 + ), where R0 is the resistance at 0 C. For tungsten filaments found in most lamps R0 = 0.50 . Take the temperature coefficient of resistance of tungsten, , to be = 0.005 K–1.  is the temperature of the filament in C.

- Calculate the temperature  and convert this temperature into kelvin.

The power dissipated in the lamp is P = VI.

- Calculate the power dissipated in the lamp at different temperatures.

- If P = kTn, what should be plotted to give a straight line?

- How is n determined?

- What should n be if the Boltzmann law is satisfied?

A graph of power P versus temperature T (in kelvin) should show two behaviours:

- an initial linear behaviour

- then a non-linear behaviour. A sample graph is shown as Graph 3.

The value for n that you got is probably smaller than what you expected.

- How is this explained?

- How can you use your version of sample graph 2 to improve the calculation of n?

- What is the improved value of n?

Graph 1 Typical voltage versus current graph for a tungsten filament lamp.

Graph 2 Power versus temperature graph for the tungsten filament lamp. Notice the changed behaviour at above 1000 K.

The change in behaviour is due to the fact that at about 1000 K the dominant mechanism of heat loss by the filament changes from convection to radiation.

- The rate of heat loss by convection is proportional to the difference in temperature between the filament and the room, TR, i.e. ,

- If radiation is the dominant mechanism then.

- In both cases  and so to a very good approximation  for convection and  for radiation.

Note: even above 1000 K where radiation is dominant, convection is still present!

The information provided here should give lots of clues for the Conclusion and Evaluation sections of your report.

Graph 3 ln P versus ln T. The slope of this graph is 3.0.