# Extension Worksheet – Topic 8, Worksheet 1

1	Outline how interference is used to read information stored on a CD.	[3]

2	Calculate the pit depth of a CD in which the wavelength of light used to read the CD is 640 nm, explaining your work.	[2]

3	Information is stored on a CD at a rate of N samples per second. Each sample consists of 32 bits. The playing time of the CD is T seconds. Calculate the storage capacity of the CD, in bits.	[2]

4	The quantum efficiency of CCD pixel is 80%. The number of photons incident on the pixel per second is 6.8 × 105. Determine the number of electrons emitted during a time interval of 120 ms.	[2]

5	The charge deposited in a pixel of capacitance 15 pF is 3.6 × 10–14 C. Calculate the voltage at the ends of the pixel.	[1]

6	The collecting area of a CCD is 62 mm2 and contains 8.0 × 106 pixels. The capacitance of a pixel is 22 pF and the quantum efficiency of the CCD is 85%.

a	Define quantum efficiency.	[1]

b	Light of intensity  and wavelength  is incident on a pixel of the CCD. Calculate the potential difference that will develop at the ends of the pixel after a time of 35 ms.	[6]

c	For the CCD in b calculate the smallest distance between two points on an object that can just be resolved given that the magnification of the CCD is 2.5 × 10–3.	[3]