# Mark scheme for Support Worksheet – Topic F, Worksheet 1

1	The modification to a carrier wave so that it can carry information.	[1]

2	In AM the amplitude of the carrier wave is modified; by the addition of the information wave displacement to the carrier amplitude.	[2]

3	In FM the amplitude of the carrier stays the same; but the frequency gets modified by an amount related to the information signal displacement.	[2]

4	The carrier frequency is 50 kHz; the information signal frequency is 5.0 kHz and the bandwidth is 10 kHz.	[3]

5		[1]

6		[1]

7	Information can be encrypted; digital signals can be perfectly regenerated.	[2]

8	A way to transmit many different signals along the same transmission line.	[1]

9	X is an analogue to digital converter; takes in an analogue signal and turns it into a binary word consisting of a specific number of bits; Y is a serial to parallel converter; once all the bits have arrived one after the other along the transmission line, the serial to parallel converter puts these bits into a single binary word.	[4]

10	The angle of incidence for which the angle of refraction is .	[1]

11	 and so 	[1]

12	(Dispersion is the general phenomenon in which different rays take different times to arrive at their destination.) Material dispersion is due to different wavelengths having different speeds; whereas modal dispersion is due to rays following different paths in a fibre.		[2]

13	Attenuation is the loss of power in a signal as it travels in a medium.	[1]

14	Noise is unwanted power/signal that is present along with the desired signal.	[1]

15	The main source of noise in an optic fibre is the dark current in the photodiode of the fibre.			[1]

16	It has infinite resistance between its two inputs; the open loop gain is infinite.	[2]

17	The output voltage has opposite sign to that of the input voltage.	[1]

18	Phone sends signal identifying itself; cellular exchange assigns phone to base station where signal is strongest; a frequency is assigned to the phone.	[3]

19	As the passenger moves, the cellular exchange computers monitor the signal strength and will automatically reroute the call to another base station in a other cell; so that the signal strength is maintained.	[2]