# Mark scheme for Extension Worksheet – Option I, Worksheet 1

1	a	The intensity of sound at eardrum is  ; hence 	[2]

b	The sound intensity level is at the threshold of pain/very high; damage will occur.	[2]

2	 and  hence ; simplifying ; and so 	[3]

3	See curve in red.

[2]

4	a	The cochlea.	[1]

b	Ordinary speech contains more high frequencies than lower frequencies; hence loss at high frequencies affects speech recognition.	[2]

5	a	; 	[2]

b	The calculation in a shows that very little ultrasound gets transmitted into tissue; placing gel on the skin makes the mismatch of impedances much less since gel and tissue have very similar impedances hence lots of ultrasound gets transmitted.	[2]

6	Attenuation of ultrasound is frequency dependent and increases with increasing frequency; so there is a limit to frequency so that attenuation does not become dominant.		[2]

7	a	radio		[1]

b	The presence of the external magnetic field means that the energy of a proton in hydrogen atoms can be high or low depending on whether the spin of the proton is aligned or not with the magnetic field; the photons are emitted in the transition of a proton from the spin down to the spin up state.	[2]

8	The patient is put in the presence of a large magnetic field; protons in the body have different energies depending on whether the spin of the proton is parallel or anti-parallel to the direction of the magnetic field; radio frequency EM waves are directed at the patient which force protons to make transitions from the spin up to the spin down state; the protons immediately make transitions down to the spin up state emitting radio frequency photons; an additional non-uniform magnetic field the patient is exposed can be used to determine the position in the body where these photons have been emitted.	[5]

9	X-rays are directed through a horizontal slice of the patient’s body; the amount of X-ray absorption at each pixel on the slice is measured so an image of the slice can be made; the procedure is repeated for other slices in the body and a computer reconstructs a 3-dimensional image; that can be viewed from any angle as the computer rotates the image.	[4]

10	a	Radiation is called ionising because (for sufficiently high energies) the radiation can knock electrons off atoms in the matter the radiation is travelling through.	[1]

b	Ionising radiation can completely kill a cell; or it can damage it to such a degree that it does not function properly; or it cannot reproduce correctly.	[3]

c	The effects of radiation are immediately obvious only for very large doses; small doses result, mainly, in long-term effects.	[2]

11	a	Photoelectric effect/Compton scattering.	[1]

b	The intensity at the tissue – bone boundary will be ; the transmitted intensity is then 	[2]

c	There is considerable difference in the intensity of X-rays before and after the bone; so that sufficient contrast exists for a image to be made.	[2]

d	X-rays take longer to expose photographic film than visible light; so X-rays that have gone through the patient are incident on a screen that emits visible light when exposed to X-rays; the visible light emitted is proportional to the intensity of X-rays and is used to expose the film.	[3]